List of parts included in kit:

| 9732-348-001 |  | KIT - BOTTOM EXHAUST OUTLET, T-80 |  |
| --- | --- | --- | --- |
| ITEM | P/N | DESCRIPTION | QTY |
| 1 | 9810-020-001 | TEE ASSEMBLY - RECIRCULATION | 1 |
| 2 | 9109-122-004 | PIPE - DUCT, 6"x17-1/2" | 1 |
| 3 | 9109-122-005 | PIPE - DUCT, 6"x23-1/2" | 2 |
| 4 | 9142-039-002 | ELBOW - 90 DEGREE AJD, 6" | 1 |
| 5 | 9545-008-003 | Screw-hxwshd,10-16x1/2,tek | 16 |
| 6 | 8507-500-001 | INSTRUCTIONS - T-80 BOTTOM EXHAUST OUTLET KIT | 1 |

*this kit also uses (1) 9142-039-002 ELBOW - 90 DEGREE AJD, 6" that is on the dryer from the factory.

Tools and supplies required:

- Drill with 1/8” bit (for drilling out rivets)

- Impact drive or hand drill with 5/16” driver bit (if using self tapping screws to install instead of rivets)

- Roll of foil duct tape

Steps:

- Remove cabinet top panel

- Remove factory recirculation duct (may require drilling out rivets)

- Remove factory exhaust outlet (may require drilling out rivets).

- Install 9810-020-001 TEE ASSEMBLY –       RECIRCULATION     with (2) provided screw (if allowable by code) into the top.

- Pre-assemble upper duct as shown.

(1) 9109-122-004 PIPE – DUCT, 6” x 17-1/2”

(2) 9142-039-002 ELBOW - 90 DEGREE AJD, 6"

(4) 9545-008-003 Screw-hxwshd,10-16x1/2,tek

Assemble elbow outlets 90° from each other. Then secure each joint with (2) screws and foil duct tape.

- Assemble duct onto machine as shown with (2) screws through the burner housing recirculation inlet into the recirculation duct.

- The machine should look like the picture at this stage.

- Pre-assemble duct with (2)  screws and foil duct tape.

(2) 9109-122-005 PIPE - DUCT, 6"x23-1/2"

- Install the duct from step 7 as shown  and put (2) screws in the upper and lower joints.

- Finish upper joints with foil duct tape.

- Finish lower joints with foil duct tape.

- Install top panel.